EdgeASIC v4.8
RTL Implementation and Verification Specification

Primary-Target Host-Assisted INT8 Conv/GEMM and YOLOv8n Compute-Layer Accelerator

| Field | Definition |
| --- | --- |
| Project identity | Host-assisted INT8 CNN compute accelerator with YOLOv8n compute-layer primary targets |
| Document purpose | Defines RTL module contracts, implementation order, verification architecture, test matrix, assertions, evidence package, and integration gates. |
| Primary hardware targets | Conv/GEMM, Conv2D windowing, 1x1/3x3 Conv, stride/padding, residual ADD, partial tiles, Tensor Table, descriptor ring, graph scheduler, split, concat, upsample, maxpool/SPPF, LUT-SiLU, detect-head convolution, raw tensor export. |
| Host software boundary | Image preprocessing, driver orchestration, CPU-only comparison, final YOLO decode, NMS, and box drawing remain host-side. |
| Version | v4.8 |
| Status | Implementation and verification specification; no RTL source file generated by this document. |

Scope rule: This document intentionally treats the previously labelled future blocks as primary targets. The verification matrix therefore includes TC-PRI test groups for each such block. Decode and NMS remain host-side software in this version because they are post-processing functions, not the primary hardware acceleration target.

# Table of Contents

1. Scope, claims, and implementation discipline

2. RTL module hierarchy and integration strategy

3. Global packages, parameters, and coding rules

4. CSR block, descriptor path, Tensor Table, and graph scheduler

5. DMA, local SRAM/BRAM, and Bank State Controller

6. Baseline Conv/GEMM datapath implementation contract

7. Primary-target operator engines

8. Host/IP synchronization and software driver contract

9. Reset, error handling, interrupts, and recovery

10. Performance counters and evaluation instrumentation

11. Verification environment architecture

12. Current-use and primary-target test matrix

13. Assertion plan

14. Regression strategy and evidence package

15. Bring-up sequence and integration gates

16. Risk register and fallback rules

Appendix A. Module checklist

Appendix B. Descriptor and CSR summary

Appendix C. Verification evidence checklist

# 1. Scope, Claims, and Implementation Discipline

## 1.1 Project definition

EdgeASIC v4.8 is defined as a host-assisted INT8 inference accelerator. The hardware IP receives prepared INT8 tensor tiles, weights, bias and quantization parameters, executes the supported compute operation, and returns output tensor tiles or raw detection tensors to the host. The host remains responsible for image preprocessing, tensor tiling, descriptor submission, unsupported software functions, final YOLO decode, NMS, and box drawing.

## 1.2 Primary target classification

| Class | Meaning | Included items |
| --- | --- | --- |
| Baseline current RTL | Must pass before any primary target is claimed | CSR, descriptor manager, dispatcher, BSC, DMA abstraction, SRAMs, 8x8 systolic array, SDDU, accumulation, requantization, ReLU/passthrough, output packing, performance counters, sticky errors. |
| Primary target extension | Included in project target and verification matrix | Conv2D window generator, 1x1/3x3 Conv, stride/padding, partial tile masking, residual ADD, Tensor Table, descriptor ring, graph scheduler, split, concat, upsample, maxpool/SPPF, LUT-SiLU, detect-head convolution, raw tensor export. |
| Host software boundary | Required for full demo but not RTL hardware block | Image preprocessing, INT8 quantization, descriptor generation, CPU-only comparison, final YOLO decode, confidence filtering, NMS, drawing boxes. |
| Explicit non-claims | Must not be stated unless separately proven | No measured FPS, no Fmax, no FPGA timing closure, no production DDR integration, no hardware decode/NMS, no autonomous full YOLOv8n hardware execution unless every graph block and post-process block is implemented and verified. |

## 1.3 Implementation discipline

Every RTL module must have a directed unit test before top-level integration.

Every cross-module valid/ready, bank ownership, descriptor commit, DMA completion, and metadata alignment rule must have an assertion or monitor.

Primary target features must be tagged with TC-PRI test IDs. A primary target feature without passing TC-PRI tests is documentation-only in the final defense.

The baseline Conv/GEMM path is the non-negotiable minimum FYP deliverable. Primary target extensions cannot break this baseline.

Host/IP data layout is part of the RTL contract. Byte ordering, tensor layout, row padding, descriptor fields, and quantization format must be fixed before DMA router RTL is frozen.

## 1.4 Version-to-version correction

The v4.8 approach replaces the older fragmented documentation set with a consolidated implementation and verification specification. Blocks that were formerly labelled future scope are now included as primary targets and assigned implementation and test responsibilities. The host-assisted model remains the central architecture identity.

# 2. RTL Module Hierarchy and Integration Strategy

Figure 1. RTL module hierarchy for the primary-target build.

## 2.1 Top-level integration shell

| Item | Requirement |
| --- | --- |
| Module | edgeasic_top |
| Purpose | Integrates CSR/control, memory movement, compute datapath, primary-target engines, error logic, performance counters, and external memory abstraction. |
| External interfaces | Clock/reset, CSR bus, memory/DMA abstraction interface, interrupt output, optional simulation debug ports. |
| Internal reset output | Synchronous reset distributed to FSMs, SRAM wrappers, FIFOs, counters, and datapath valid registers. |
| Integration rule | No engine can start directly from host CSRs. All starts pass through descriptor manager, descriptor ring, graph scheduler, and operator dispatcher. |

## 2.2 Ownership split

| Owner area | Responsibility |
| --- | --- |
| Datapath owner | PE, array, SDDU, accumulation engine, requantization, activation, residual ADD, partial tile mask, Conv2D window generator, LUT-SiLU, maxpool/SPPF datapath, detect-head compute datapath. |
| Control/memory owner | CSR, descriptor manager, descriptor ring, Tensor Table, graph scheduler, dispatcher, BSC, DMA, SRAM wrappers, packing FIFO, prefetch/compute/writeback FSMs. |
| Shared verification owner | Both designers write tests for their own blocks. The shared regression harness, scoreboard, and Python golden-model API are jointly reviewed weekly. |
| Joint integration weeks | Top-level integration, accumulation RMW, residual SRAM/BSC interaction, graph scheduler, and CPU+IP demo are joint tasks. |

## 2.3 Integration order

Packages/types -> CSR/descriptor -> SRAM/BSC/DMA -> PE array -> SDDU -> accumulation -> requantization -> activation/elementwise -> output packing -> top-level baseline -> performance counters/errors -> primary operator engines -> graph/tensor sequencing -> host-assisted demo.

Each arrow is a gate. A downstream module may use a stub of an upstream module for early unit testing, but cannot be declared integrated until the real upstream module passes its test ID group.

The first end-to-end RTL milestone is a single Conv/GEMM tile. Primary-target operators come after the baseline end-to-end test passes.

# 3. Global Packages, Parameters, and Coding Rules

## 3.1 Required packages

| Package | Required content |
| --- | --- |
| config_pkg.sv | Global architectural constants, enum encodings, tile dimensions, SRAM widths, AXI widths, CSR offsets, default reset values. |
| types_pkg.sv | Signed arithmetic typedefs, packed metadata structs, descriptor structs, Tensor Table structs, status/counter structs. |
| quant_pkg.sv | Requantization widths, scale/shift rules, saturation functions, activation mode limits. |
| assert_pkg.sv | Reusable SVA macros or checker modules for valid/ready stability, one-hot states, metadata alignment, and no-deadlock timers. |

## 3.2 Numeric widths

| Quantity | Width | Implementation rule |
| --- | --- | --- |
| Activation | signed INT8 | Two-complement input tensor elements. Symmetric INT8 is the recommended minimum path. |
| Weight | signed INT8 | Two-complement weight values. Per-channel quantization supported through BQ SRAM parameters. |
| Product | signed INT16 | INT8 x INT8 product. |
| Accumulator | signed INT32 | MAC accumulation across K dimension. |
| Bias sum | signed INT33 | INT32 accumulator plus INT32 bias requires one extra sign bit. |
| Scale | signed INT24 | Per-output-channel quantization multiplier. |
| Scaled product | signed INT57 | INT33 x INT24. |
| Residual intermediate | signed INT9 | INT8 output plus INT8 residual before saturation. |
| Output | signed INT8 | Saturated to [-128, 127]. |

## 3.3 Dynamic pipeline metadata

Required pipe_meta_t fields

typedef struct packed {
    logic                 valid;
    logic                 k_tile_first;
    logic                 k_tile_last;
    logic [ACC_ADDR_W-1:0] acc_addr;
    logic [CHAN_W-1:0]     out_chan_base;
    logic [LANES-1:0]      lane_valid;
    logic [TENSOR_ID_W-1:0] dst_tensor_id;
} pipe_meta_t;

Non-negotiable rule: K_TILE_FIRST and K_TILE_LAST are dynamic metadata generated by the compute/control FSM for each K-subtile. They are not static descriptor flags and must not be written by host software as descriptor bits.

## 3.4 Coding rules

All arithmetic on INT8, INT16, INT32, INT33, and INT57 values must be declared signed at the signal level or explicitly cast before arithmetic.

Requantization right shift must use arithmetic shift: $signed(scaled_product) >>> shift_amount.

No inferred latches. All FSM next-state logic must have defaults for every output and next state.

All valid payloads must remain stable while valid=1 and ready=0.

Asynchronous resets are avoided unless required by the selected FPGA board. The default RTL coding style is synchronous active-high reset.

Primary-target engines may be disabled by synthesis parameter, but if disabled their OP_TYPE must produce a sticky unsupported_op_error rather than hanging.

No block may silently drop a descriptor. It must either accept, stall with backpressure, or raise a defined error.

# 4. CSR Block, Descriptor Path, Tensor Table, and Graph Scheduler

## 4.1 CSR register map

| Offset | Register | Access | Definition |
| --- | --- | --- | --- |
| 0x00 | CONTROL | RW | START, SOFT_RESET, PERF_CLEAR, IRQ_ENABLE, descriptor_commit |
| 0x04 | STATUS | RO/W1C | DONE, BUSY, ERR_UNSUPPORTED_OP, ERR_UNSUPPORTED_ACT, ERR_AXI_BOUNDARY, ERR_SHAPE, ERR_DMA_RESP, ERR_TIMEOUT |
| 0x08 | IRQ_STATUS | RO/W1C | irq_done, irq_error, irq_perf |
| 0x0C | OP_CONFIG | RW | OP_TYPE, ACT_MODE, EW_MODE, engine enable bits |
| 0x10-0x3C | Descriptor window | RW | A_BASE, W_BASE, O_BASE, BQ_BASE, INPUT1_BASE, M, N, K, strides, flags, tensor IDs |
| 0x40-0x7C | Tensor Table access window | RW | tensor_id, base, shape, stride, scale id, producer, consumer, valid |
| 0x80 | PERF_TOTAL_CYCLES | RO | Cycles from START to DONE or ERROR. |
| 0x84 | PERF_COMPUTE_CYCLES | RO | Cycles where compute datapath is active. |
| 0x88 | PERF_DMA_WAIT_CYCLES | RO | Cycles stalled waiting for DMA or bank valid. |
| 0x8C | PERF_WRITEBACK_CYCLES | RO | Cycles spent packing or writing output. |
| 0x90 | PERF_STALL_CYCLES | RO | Pipeline hold or backpressure cycles. |
| 0x94 | PERF_MAC_ACTIVE_CYCLES | RO | Cycles where PE array performs valid MACs. |
| 0x98 | PERF_LAYER_COUNT | RO | Completed descriptors/layers. |
| 0x9C | PERF_ERROR_COUNT | RO | Sticky error event count. |

## 4.2 Descriptor fields

| Descriptor field group | Implementation requirement |
| --- | --- |
| op_type | Selects Conv/GEMM, Conv2D, split, concat, upsample, maxpool, SPPF, LUT-SiLU, C2f sequence, detect-head conv, raw tensor export. |
| act_mode | ACT_NONE, ACT_RELU, ACT_LUT_SILU. Unsupported modes latch ERR_UNSUPPORTED_ACT. |
| ew_mode | EW_NONE or EW_ADD for residual addition. |
| base addresses | A_BASE, W_BASE, O_BASE, BQ_BASE, INPUT1_BASE; can be direct or resolved through Tensor Table. |
| shape fields | M, N, K plus H, W, C where Conv2D/tensor ops require spatial interpretation. |
| stride/kernel/padding | kernel_h, kernel_w, stride_h, stride_w, pad_top, pad_bottom, pad_left, pad_right. |
| flags | ROW_PAD_EN, RESIDUAL_EN, PARTIAL_TILE_EN, PER_CH_QUANT_EN, TENSOR_TABLE_EN, GRAPH_CONTINUE_EN. |
| tensor IDs | src0_tensor_id, src1_tensor_id, weight_tensor_id, dst_tensor_id, quant_tensor_id. |
| static only rule | Descriptor flags are static layer-level options. K_TILE_FIRST/LAST are generated dynamically and travel as pipe_meta_t fields. |

## 4.3 Descriptor commit behavior

Host writes inactive descriptor window while engine may be idle or busy.

Host asserts descriptor_commit. Descriptor manager checks legality, shape, alignment, supported OP_TYPE, and available inactive slot.

If accepted, commit_accept=1 for one cycle and the inactive descriptor becomes eligible for graph scheduler or direct dispatch.

If busy and no inactive slot exists, commit_accept=0. The current active descriptor must not change.

If descriptor is illegal, commit_accept=0 and ERR_SHAPE or relevant error bit latches.

Software recovery after error: read STATUS, optionally read debug registers, clear W1C error bits, re-write a legal descriptor, then START again. Automatic restart is forbidden.

## 4.4 Tensor Table

| Topic | Requirement |
| --- | --- |
| Purpose | Stores tensor metadata so the graph scheduler can chain layers without host rewriting every address. |
| Minimum entries | At least 16 entries for FYP prototype. Parameterizable upward. |
| Fields | valid, base_addr, H, W, C, stride_H, stride_W, row_pad_64B, scale_id, zero_point, producer_desc, consumer_count, layout_id. |
| Baseline direct mode | If TENSOR_TABLE_EN=0, engine uses descriptor base addresses directly. |
| Graph mode | If enabled, graph scheduler resolves src/dst tensor IDs to actual base/shape/stride fields. |
| Verification | Tensor Table must have direct-mode ignore test, read/write test, invalid tensor ID test, and two-layer chain test. |

## 4.5 Graph scheduler

| Scheduler rule | Requirement |
| --- | --- |
| Scheduling model | In-order descriptor ring execution is the primary target. Out-of-order issue is not required. |
| Dependency model | A descriptor can start when all source tensor entries are valid and required banks are available. |
| Completion model | On layer completion, dst_tensor_id is marked valid and producer field is updated. |
| Unsupported dependency | If a descriptor refers to an unsupported op or invalid tensor, scheduler stalls only if data is pending; otherwise it raises sticky error. It must not hang forever. |
| Host interaction | Host may run graph mode or single-descriptor mode. The demo can use host-managed layer sequencing if graph scheduler is incomplete, but then graph scheduler cannot be claimed as verified RTL. |

# 5. DMA, Local SRAM/BRAM, and Bank State Controller

Figure 2. Compute pipeline and primary-target insertion points.

## 5.1 Memory hierarchy

| Memory block | Implementation requirement |
| --- | --- |
| External memory abstraction | AXI memory model or board-level DDR bridge. Real production DDR integration is not automatically claimed. |
| Activation SRAM | Ping-pong INT8 tile banks. Current bank consumed by compute while next bank may be prefetched. |
| Weight SRAM | Ping-pong INT8 weight banks. Weight-stationary PE loading uses selected bank. |
| Bias/Quant SRAM | Stores per-channel bias, scale, shift, and optional zero-point metadata. |
| Accumulation buffer | INT32 partial sums used for K-tile reductions. Must support deterministic read-modify-write. |
| Residual SRAM | Primary target input bank for EW_ADD residual path. |
| Output SRAM | INT8 output tile staging before packing FIFO and writeback. |
| Tensor Table | Metadata store; not tensor payload store. |

## 5.2 AXI DMA boundary and response policy

DMA requests crossing a 4KB boundary are locally rejected. The DMA abstraction shall not assert external ARVALID/AWVALID/WVALID for the rejected request.

On rejection, the DMA abstraction asserts dma_done=1 and dma_error=1 and causes STATUS.ERR_AXI_BOUNDARY to latch.

Requesting FSM treats the request as completed with error and exits its wait state. Deadlock on rejected bursts is forbidden.

Hardware burst splitting is not required in the primary build; software must pre-align descriptors and tile buffers to avoid 4KB crossings.

AXI response errors such as SLVERR/DECERR latch ERR_DMA_RESP and complete the request with error.

## 5.3 DMA read router byte ordering

Mandatory byte-ordering contract

AXI beat byte lane definition:
beat[7:0]      -> byte index 0 at lowest memory address
beat[15:8]     -> byte index 1
...
beat[511:504]  -> byte index 63 at highest address within the 64-byte beat

SRAM lane mapping rule:
SRAM_WORD[element_index] = beat[8*element_index +: 8]
for element_index = 0..63 unless a descriptor-specific tensor layout remap is explicitly selected.

## 5.4 Bank State Controller states

| State | Meaning |
| --- | --- |
| INVALID | Bank contents are not usable. Reset state. |
| FILLING | DMA/router is writing bank. Compute cannot consume. |
| VALID | Bank contains a complete tile and may be consumed by a matching engine. |
| CONSUMING | Compute or operator engine owns the bank for read. |
| CONSUMED | Consumer finished. Bank can be refilled or invalidated. |
| ERROR_LOCKED | Bank was involved in an error. Requires reset or explicit host recovery. |

## 5.5 BSC invariants

A bank cannot transition from FILLING to CONSUMING without passing through VALID.

A bank cannot be written while in CONSUMING.

VALID/CONSUMED handshake is one ownership transfer per tile.

Residual SRAM follows the same ownership rules as activation and weight SRAM when EW_ADD is enabled.

No FSM may wait forever for a bank without a timeout assertion in simulation.

# 6. Baseline Conv/GEMM Datapath Implementation Contract

## 6.1 Weight-stationary PE contract

| PE topic | Requirement |
| --- | --- |
| Dataflow | Weight-stationary. Weights are loaded and held in PE-local registers while activations propagate through the array. |
| Arithmetic | signed INT8 activation x signed INT8 weight -> signed INT16 product -> signed INT32 partial-sum update. |
| act_out | Registered one-cycle activation propagation output. Systolic timing assumes this register exists. |
| Weight load | Weight load enable is independent from MAC valid. MACs are only valid after weight load completion. |
| Reset | Clears valid state and local accumulator/control state. Weight data after reset is invalid until explicitly loaded. |
| Test requirement | Signed positive/negative tests, act_out delay test, weight-hold test, array-level timing test. |

## 6.2 SDDU implementation

The Systolic Drain Deskew Unit aligns diagonal systolic outputs into rectangular channel-major output timing. For N=8, lane 0 is delayed by 7 cycles, lane 1 by 6 cycles, and lane 7 by 0 cycles. The same delay is applied to pipe_meta_t so that data, valid, K-tile metadata, accumulator address, output-channel base, and lane_valid mask remain aligned.

SDDU timing rule

for lane i in 0..N-1:
    delay_cycles[i] = N - 1 - i
    data_out[i]     = delay_line_data[i][delay_cycles[i]]
    meta_out[i]     = delay_line_meta[i][delay_cycles[i]]

stall rule:
    if downstream_ready == 0:
        all SDDU delay lines hold both data and metadata together

## 6.3 Accumulation engine

| K-tile case | Required behavior |
| --- | --- |
| FIRST=1, LAST=1 | Single K-tile layer. Bias + current MAC sum -> requantization/output path. No partial buffer dependency. |
| FIRST=1, LAST=0 | First subtile. Bias + MAC sum -> write partial INT32 accumulator. Do not produce final output. |
| FIRST=0, LAST=0 | Middle subtile. Read previous partial, add MAC sum, write updated partial. Do not produce final output. |
| FIRST=0, LAST=1 | Final subtile. Read previous partial, add MAC sum -> requantization/output path. Final partial writeback not required unless debug mode enabled. |

Verification warning: The accumulation engine is the most correctness-sensitive datapath block. Bias must be added exactly once per output element. K_TILE_LAST controls final drain into requantization; non-last subtiles must not update output SRAM.

## 6.4 Requantization and activation

Requantization and activation intent

// Required arithmetic intent, not full RTL source
biased_sum      = signed_int33(accumulator_int32 + bias_int32)
scaled_product  = signed_int57(biased_sum * scale_int24)
shifted         = $signed(scaled_product) >>> shift_amount
saturated_int8  = clamp(shifted, -128, 127)

ACT_NONE: output = saturated_int8
ACT_RELU: output = max(saturated_int8, 0)
ACT_LUT_SILU: output = lut_silu(saturated_int8) when enabled and verified

## 6.5 Partial tile and residual path

| Feature | Implementation rule |
| --- | --- |
| Partial tile mask | Lane-valid mask suppresses MAC contribution, accumulator writes, output SRAM writes, and performance MAC-active counting for invalid lanes. |
| Residual ADD | If EW_ADD and FLAG_RESIDUAL_EN are set, residual SRAM must be prefetched before elementwise unit consumption. |
| Residual arithmetic | INT8 output + INT8 residual -> signed INT9 -> saturate to INT8. |
| Residual shape rule | Residual tensor shape must match output tile shape. Mismatches raise ERR_SHAPE. |
| Residual quantization rule | Minimum path assumes host pre-aligns residual quantization. Hardware does not rescale residual unless explicitly added in a later revision. |

# 7. Primary-Target Operator Engines

## 7.1 OP_TYPE map

| OP_TYPE | Class | Meaning |
| --- | --- | --- |
| OP_CONV_GEMM | Baseline current | Dense tiled INT8 GEMM / lowered convolution. |
| OP_CONV2D | Primary target | Conv2D window generation feeding baseline systolic array. |
| OP_SPLIT | Primary target | Tensor split/view generation. |
| OP_CONCAT | Primary target | Tensor channel/tile merge. |
| OP_UPSAMPLE | Primary target | Nearest-neighbor upsample by repeated address pattern. |
| OP_MAXPOOL | Primary target | Windowed max reduction. |
| OP_SPPF | Primary target | Repeated maxpool + concat-style tensor path. |
| OP_LUT_SILU | Primary target | LUT-based SiLU approximation. |
| OP_C2F_SEQ | Primary target | C2f primitive sequence controller. |
| OP_DETECT_CONV | Primary target | Detect-head convolution mapping. |
| OP_RAW_EXPORT | Primary target | Write raw detection tensor format for host decode/NMS. |

## 7.2 Conv2D window generator

Accepts tensor shape H, W, C and descriptor fields kernel_h, kernel_w, stride_h, stride_w, and padding.

1x1 Conv mode may bypass spatial window buffering and feed GEMM directly with channel-major tiles.

3x3 Conv mode produces a serialized K dimension equal to kernel_h * kernel_w * input_channels.

Zero padding generates zero-valued activation lanes without reading out-of-range memory.

Window generator must preserve the same pipe_meta_t conventions used by the baseline datapath.

The first implementation target is fixed 1x1 and fixed 3x3 stride-1 padding cases; generalized stride/padding is verified after basic cases pass.

## 7.3 Tensor split, concat, and upsample engines

| Engine | Implementation rule |
| --- | --- |
| Split | Creates multiple output tensor views or writes channel ranges to separate destination tensors. Test with exact channel boundary, odd channel boundary, and partial tile boundary. |
| Concat | Merges input tensors along the channel dimension. Requires shape compatibility except concat axis. Must verify byte layout and row padding. |
| Upsample | Nearest-neighbor repeat in H/W dimensions. Address generator reads each source element multiple times and writes expanded tensor shape. |
| Routing | These engines may use DMA/memory movement path rather than PE array. They still use descriptor, Tensor Table, BSC, and output SRAM protocols. |
| Error behavior | Invalid shapes, unsupported axes, and incompatible row padding raise ERR_SHAPE. No silent output corruption allowed. |

## 7.4 Maxpool and SPPF

| Block | Rule |
| --- | --- |
| Maxpool | Compares INT8 values over a local window. First target is 2x2 or 3x3 window depending on selected model subset. Padding behavior must match Python golden model. |
| SPPF | Implemented as repeated maxpool and concatenation-style output composition. It may be sequenced by graph scheduler or an SPPF micro-sequencer. |
| Verification | Maxpool isolated tests must pass before SPPF tests. SPPF tests must compare exact tensor shape and byte layout, not only numerical values. |

## 7.5 LUT-SiLU activation

| Topic | Requirement |
| --- | --- |
| Input | Signed INT8 activation value after requantization or pre-activation intermediate if selected. |
| Implementation option A | 256-entry INT8 LUT indexed by input value offset by 128. |
| Implementation option B | Piecewise approximation with small LUT segments. Only allowed if Python golden model uses identical approximation. |
| Output | Signed INT8 activation result. |
| Enable rule | ACT_LUT_SILU is primary target. If block is disabled by parameter, selecting it must raise ERR_UNSUPPORTED_ACT. |
| Test rule | Compare all 256 INT8 input values against Python LUT reference. |

## 7.6 C2f and detect-head path

| Feature | Implementation rule |
| --- | --- |
| C2f sequencer | Sequences primitive Conv, split, optional residual/add, and concat operations. It is a controller/micro-sequencer, not a new MAC datapath. |
| Detect-head convolution | Maps detect-head conv layers to Conv/GEMM or Conv2D path and writes raw detection tensor format. |
| Raw tensor export | Output tensor must be written in a host-readable layout with documented shape, stride, scale, and channel meaning. |
| Host boundary | YOLO decode, confidence thresholding, and NMS consume raw tensor on host. Hardware does not draw boxes. |

# 8. Host/IP Synchronization and Software Driver Contract

## 8.1 Host responsibilities

Image load, resize/letterbox, normalization, and INT8 quantization.

Tile and weight packing using the documented NHWC-like layout and 64-byte row padding.

Descriptor generation and CSR writes.

Polling DONE/ERROR or handling interrupt.

Output tensor reconstruction and comparison against golden model.

Decode, confidence filtering, NMS, and final box drawing in the YOLO demo.

CPU-only baseline timing for demonstration comparison.

## 8.2 IP responsibilities

Accept legal descriptors and reject illegal descriptors with sticky CSR errors.

Move input tiles to local SRAM/BRAM via DMA abstraction.

Execute supported compute and tensor operations.

Expose CSR performance counters for kernel-level evaluation.

Return output tensor tiles or raw tensors in documented byte layout.

Never hang silently on unsupported OP_TYPE, unsupported activation, 4KB boundary violation, invalid shape, or DMA response error.

## 8.3 CPU-only versus CPU+IP demo measurement

| Measurement | Definition |
| --- | --- |
| CPU-only kernel | CPU runs same INT8 Conv/GEMM or layer reference. Used as software baseline. |
| IP kernel | IP performance counters measure compute cycles, DMA wait, stalls, and writeback cycles. |
| Full system CPU-only | CPU preprocess, compute, decode/NMS, and display. |
| Full system CPU+IP | CPU preprocess, transfer/tiling, IP compute, transfer back, CPU decode/NMS/display. |
| Expected result | Kernel speedup is the primary hardware claim. Full-system speedup depends on transfer bandwidth and should be reported separately. |

# 9. Reset, Error Handling, Interrupts, and Recovery

## 9.1 Reset behavior

| Block | Reset requirement |
| --- | --- |
| CSR registers | Reset CONTROL to 0, STATUS to idle/no-error, performance counters to 0 unless debug preserve is enabled. |
| Descriptor manager | Active/inactive descriptors invalidated. |
| Descriptor ring | Head/tail reset to 0, entries invalid. |
| Tensor Table | Entries invalid unless loaded by host after reset. |
| BSC | All banks INVALID. |
| FIFOs | Empty. |
| Datapath | Valid bits cleared, internal hold registers invalid. |
| FSMs | IDLE state only. |

## 9.2 Sticky error bits

| Error bit | Meaning |
| --- | --- |
| ERR_UNSUPPORTED_OP | Unsupported or disabled OP_TYPE selected. |
| ERR_UNSUPPORTED_ACT | Unsupported or disabled activation selected. |
| ERR_AXI_BOUNDARY | DMA request crosses 4KB boundary and is locally rejected. |
| ERR_DMA_RESP | AXI/memory response error. |
| ERR_SHAPE | Illegal dimensions, incompatible tensor shape, invalid stride/padding, residual mismatch, concat mismatch. |
| ERR_TIMEOUT | Simulation/watchdog timeout for an operation that failed to complete. |
| ERR_TENSOR | Invalid Tensor Table entry or dependency not satisfiable. |
| ERR_INTERNAL | Assertion-exposed internal fatal error for debug builds. |

## 9.3 Recovery sequence

Host observes IRQ_ERROR or STATUS error bit.

Host reads STATUS and optional debug registers.

Host clears W1C error bits.

Host may assert SOFT_RESET if bank/FSM recovery is uncertain.

Host rewrites legal descriptor/Tensor Table entries.

Host starts operation again. Hardware never auto-restarts from an error state.

# 10. Performance Counters and Evaluation Instrumentation

| Counter | Increment predicate |
| --- | --- |
| PERF_TOTAL_CYCLES | Increment every cycle from accepted START to DONE or ERROR. |
| PERF_COMPUTE_CYCLES | Increment when PE array or active operator engine is doing useful work. |
| PERF_DMA_WAIT_CYCLES | Increment when a compute/control FSM waits for DMA completion or valid bank. |
| PERF_WRITEBACK_CYCLES | Increment during output packing and writeback. |
| PERF_STALL_CYCLES | Increment when valid work exists but pipeline is held by backpressure. |
| PERF_MAC_ACTIVE_CYCLES | Increment when at least one valid MAC lane is active. Invalid partial lanes are excluded from utilization calculations. |
| PERF_LAYER_COUNT | Increment after each descriptor completes successfully. |
| PERF_ERROR_COUNT | Increment once per sticky error event. |

Evaluation rule: Performance reporting must show kernel-only and full host-assisted measurements separately. A slow prototype transfer path can hide kernel acceleration and must not be used to discredit the compute IP if the counter-based kernel results are strong.

# 11. Verification Environment Architecture

Figure 3. Verification environment and evidence flow.

## 11.1 Verification layers

| Verification layer | Purpose |
| --- | --- |
| Unit tests | One testbench per RTL module or tightly coupled block. Required before integration. |
| Subsystem tests | Control plane, DMA/BSC/memory, Conv/GEMM datapath, primary operator engines. |
| Integration tests | Full edgeasic_top with memory model and CSR driver. |
| Graph tests | Descriptor ring + Tensor Table + graph scheduler running two or more operations. |
| Demo tests | CPU-only versus CPU+IP flow; output comparison plus performance measurement. |
| Negative tests | Illegal descriptor, unsupported/disabled feature, 4KB crossing, DMA error, invalid tensor ID, shape mismatch. |

## 11.2 Python golden model contract

Python reference is the source of expected outputs for INT8 GEMM, Conv2D, quantization, ReLU, LUT-SiLU, maxpool, split, concat, upsample, C2f primitive sequence, detect-head mapping, and raw tensor export.

The golden model must use the same rounding/truncation, saturation, layout, padding, and stride rules as the RTL specification.

Every test vector stores seed, descriptor, input tensors, weights, BQ parameters, expected output, and tolerance policy.

For baseline INT8 arithmetic, comparison should be bit-exact. For approximate LUT-SiLU, comparison should be exact against the selected LUT, not against floating-point SiLU unless explicitly specified.

## 11.3 Scoreboard rules

Scoreboard compares output SRAM/DMA writeback data against expected byte layout.

Intermediate probes are used for debug but final pass/fail must be based on externally visible outputs and CSR status.

For errors, scoreboard verifies that the engine does not start or safely stops, sticky bits latch, and W1C clears them.

Timeout monitor fails any test that does not reach DONE or ERROR within the expected cycle budget.

# 12. Current-Use and Primary-Target Test Matrix

## 12.1 Baseline/current-use test IDs

| TC ID | Test name | Blocks | Coverage goal |
| --- | --- | --- | --- |
| TC-CUR-001 | CSR reset and W1C status | CSR/status | GOOD required |
| TC-CUR-002 | Descriptor commit legal/illegal | descriptor_manager | GOOD required |
| TC-CUR-003 | Unsupported OP_TYPE latches error | dispatcher/CSR | GOOD required |
| TC-CUR-004 | Unsupported/disabled activation error | activation/CSR | GOOD required |
| TC-CUR-005 | DMA 4KB local rejection no deadlock | DMA/FSM/CSR | GOOD required |
| TC-CUR-006 | DMA response error propagation | DMA/FSM/CSR | GOOD required |
| TC-CUR-007 | BSC legal and illegal transitions | BSC | GOOD required |
| TC-CUR-008 | PE signed arithmetic | PE | GOOD required |
| TC-CUR-009 | Registered act_out timing | PE/array | GOOD required |
| TC-CUR-010 | 8x8 systolic timing and output order | array | GOOD required |
| TC-CUR-011 | SDDU delay direction | SDDU | GOOD required |
| TC-CUR-012 | SDDU metadata alignment | SDDU | GOOD required |
| TC-CUR-013 | K_TILE_FIRST/LAST generation and four cases | compute/accum | GOOD required |
| TC-CUR-014 | Accumulator RMW hazard | accum_buffer | GOOD required |
| TC-CUR-015 | Requant signed shift negative values | requant | GOOD required |
| TC-CUR-016 | INT8 saturation boundary +127/-128 | requant | GOOD required |
| TC-CUR-017 | ReLU and passthrough | activation | GOOD required |
| TC-CUR-018 | DMA read router byte ordering | router | GOOD required |
| TC-CUR-019 | Packing FIFO backpressure | packing_fifo | GOOD required |
| TC-CUR-020 | Performance counters and PERF_CLEAR | CSR/perf | GOOD required |

## 12.2 Primary-target test IDs

| TC ID | Test name | Main blocks |
| --- | --- | --- |
| TC-PRI-001 | Partial tile masking both M and N non-multiple of 8 | mask/array/accum |
| TC-PRI-002 | Residual SRAM prefetch and EW_ADD saturation | res_sram/ewu/BSC |
| TC-PRI-003 | Tensor Table direct read/write and baseline ignore mode | tensor_table |
| TC-PRI-004 | Descriptor ring two-entry sequence | descriptor_ring |
| TC-PRI-005 | Graph scheduler two-layer dependency chain | graph_scheduler |
| TC-PRI-006 | 1x1 Conv mapped to GEMM | conv2d/win/array |
| TC-PRI-007 | 3x3 Conv stride-1 padding case | window_gen/array |
| TC-PRI-008 | General stride/padding boundary tiles | window_gen |
| TC-PRI-009 | LUT-SiLU all 256 INT8 inputs | lut_silu |
| TC-PRI-010 | Maxpool isolated window test | maxpool |
| TC-PRI-011 | SPPF primitive sequence test | sppf/maxpool/concat |
| TC-PRI-012 | Tensor split channel boundary test | split |
| TC-PRI-013 | Tensor concat layout and row padding test | concat |
| TC-PRI-014 | Nearest upsample address repeat test | upsample |
| TC-PRI-015 | C2f primitive sequence test | c2f/graph |
| TC-PRI-016 | Detect-head conv synthetic layer | detect/conv |
| TC-PRI-017 | Raw tensor export layout compatibility | raw_export/host |
| TC-PRI-018 | CPU-only vs CPU+IP kernel comparison | demo/perf |
| TC-PRI-019 | Host-side decode/NMS compatibility with raw tensor | host software |
| TC-PRI-020 | Full primary-target regression smoke suite | top-level |

## 12.3 Negative tests

Illegal descriptor dimensions M=0, K=0, H/W mismatch, concat mismatch, residual shape mismatch.

Crossing 4KB DMA request.

Invalid Tensor Table ID.

OP_TYPE disabled by build parameter.

Graph scheduler dependency that cannot be satisfied.

Stride/padding configuration outside implemented limits.

Packing FIFO forced full while writeback is active.

AXI memory model returns error response.

# 13. Assertion Plan

| Assertion ID | Property |
| --- | --- |
| A-CSR-001 | Sticky error bit remains set until W1C clear. |
| A-DESC-001 | K_TILE_FIRST/LAST are not read from descriptor flags. |
| A-DESC-002 | Active descriptor cannot change while BUSY=1. |
| A-BSC-001 | No bank is written and consumed in same cycle unless explicitly dual-port safe. |
| A-BSC-002 | VALID bank eventually becomes CONSUMED after consumer accepts it. |
| A-DMA-001 | 4KB crossing request does not issue external AXI valid. |
| A-DMA-002 | 4KB crossing request produces dma_done and dma_error. |
| A-PE-001 | act_out is registered, not combinational pass-through. |
| A-SDDU-001 | Lane delay equals N-1-i. |
| A-SDDU-002 | Data and pipe_meta_t valid are delayed identically. |
| A-ACC-001 | Non-last K tile cannot write final output SRAM. |
| A-ACC-002 | Bias is added only when k_tile_first=1. |
| A-REQ-001 | Shift amount uses arithmetic right shift on signed product. |
| A-FIFO-001 | Packing FIFO data remains stable under backpressure. |
| A-GRAPH-001 | Graph scheduler starts descriptor only when dependencies are valid. |
| A-TENSOR-001 | Invalid Tensor Table entry prevents engine start and latches error. |
| A-HOST-001 | DONE and ERROR are mutually exclusive for a single operation completion cycle unless configured otherwise. |

# 14. Regression Strategy and Evidence Package

## 14.1 Regression groups

| Regression group | Contents |
| --- | --- |
| smoke | Compile, reset, CSR read/write, one tiny GEMM. |
| current_datapath | PE, array, SDDU, accumulation, requant, activation. |
| current_control | CSR, descriptor, dispatcher, BSC, DMA, errors. |
| current_system | End-to-end Conv/GEMM tests including negative values and multi-K tiles. |
| primary_ops | Conv2D, partial tiles, residual, LUT-SiLU, maxpool, split, concat, upsample. |
| graph | Tensor Table, descriptor ring, graph scheduler, C2f primitive sequence. |
| demo | CPU-only vs CPU+IP kernel comparison, raw tensor export, host decode/NMS compatibility. |
| nightly | All deterministic tests plus randomized seeded tests. |

## 14.2 Required evidence

Regression summary table with pass/fail, seed, date, and commit ID.

Waveforms for PE/array timing, SDDU metadata alignment, K_TILE_FIRST/LAST, DMA 4KB error, BSC ownership, graph scheduler layer chain, and raw tensor export.

Python golden comparison logs with maximum absolute error and exact byte mismatch report if failing.

Performance counter dumps for kernel-only and host-assisted demo.

Feature status table: implemented and verified, implemented not fully verified, documentation-only, disabled, or not attempted.

# 15. Bring-Up Sequence and Integration Gates

| Gate | Name | Exit criterion |
| --- | --- | --- |
| Gate 0 | Spec freeze | Enums, widths, layout, CSR map, descriptor fields frozen. |
| Gate 1 | Control smoke | CSR, descriptor, dispatcher, sticky errors pass. |
| Gate 2 | Memory safety | BSC, SRAM wrappers, DMA 4KB rejection pass. |
| Gate 3 | Compute unit | PE array and SDDU pass. |
| Gate 4 | Accumulation correctness | Four K-tile cases and RMW tests pass. |
| Gate 5 | Quantized output | Requant, saturation, ReLU pass. |
| Gate 6 | First top-level tile | End-to-end Conv/GEMM tile matches Python. |
| Gate 7 | Baseline regression | Current test matrix passes. |
| Gate 8 | Optional/current primary stability | Partial/residual tests pass or are disabled without regression. |
| Gate 9 | Conv2D primary target | 1x1 and selected 3x3 tests pass. |
| Gate 10 | Graph/tensor primary target | Tensor Table, descriptor ring, two-layer graph test pass. |
| Gate 11 | YOLO-style primary demo | Selected operator sequence and raw tensor export pass host compatibility test. |
| Gate 12 | Defense package | Documentation, waveforms, logs, demos, safe claims ready. |

# 16. Risk Register and Fallback Rules

| ID | Risk | Severity | Mitigation/fallback |
| --- | --- | --- | --- |
| R1 | Person B overload | High | Both designers write tests; graph/tensor tasks become joint. |
| R2 | SDDU/act_out timing mismatch | High | Waveform review before accumulation integration. |
| R3 | K_TILE_FIRST/LAST bug | Critical | Four-case directed test and assertion before random tests. |
| R4 | Accumulation RMW hazard | Critical | Serialize first implementation if pipelined RMW fails. |
| R5 | DMA byte ordering mismatch | High | Known-pattern 0..63 byte test before top-level. |
| R6 | Graph scheduler complexity | High | Limit primary graph scheduler to in-order two/three-layer chains first. |
| R7 | Conv2D window generator complexity | High | Implement 1x1 first; then fixed 3x3; generalize only after passing. |
| R8 | SPPF/C2f too large | Medium | Implement as primitive sequence and document disabled cases clearly. |
| R9 | CPU+IP full-system speedup hidden by transfer overhead | Medium | Report kernel-only and full-system timing separately. |
| R10 | Overclaim risk | High | Final slides must include explicit non-claim statement. |

## 16.2 Fallback classification rules

If a primary-target block has RTL but no passing TC-PRI test, it must be marked implemented-but-unverified, not completed.

If a primary-target block only has architecture and Python model, it must be marked documentation/Python-only.

If a block is disabled by parameter, selecting it must raise a sticky unsupported error.

The baseline Conv/GEMM implementation and verification must not be sacrificed to chase primary-target extensions.

The final defense claim must reflect actual evidence, not plan intent.

# Appendix A. Module Checklist

| Category | Modules |
| --- | --- |
| Top/control | edgeasic_top, csr_block, descriptor_manager, descriptor_ring, tensor_table, graph_scheduler, operator_dispatcher, error_irq_logic, perf_counter_block |
| Memory | axi_dma_master_abs, dma_arbiter, dma_read_router, activation_sram, weight_sram, bias_quant_sram, residual_sram, accum_buffer, output_sram, packing_fifo, bank_state_controller |
| Baseline datapath | pe_mac, systolic_array_8x8, sddu, accum_engine, requant_pipeline, activation_unit, elementwise_unit, partial_tile_mask |
| Primary engines | conv2d_window_generator, tensor_split_engine, tensor_concat_engine, nearest_upsample_engine, maxpool_engine, sppf_engine, lut_silu_unit, c2f_block_sequencer, detect_head_conv_controller, raw_tensor_export_controller |
| Verification | tb_edgeasic_top, csr_driver, axi_memory_model, scoreboard, python_vector_loader, monitors, assertion bind files, regression scripts |

# Appendix B. Descriptor and CSR Summary

The descriptor and CSR tables in Chapters 4 and 10 are the authoritative interface summary for v4.8 implementation planning. Any RTL implementation must preserve reset values, access types, W1C semantics, and dynamic metadata separation. If a field is not yet supported by an RTL block, the dispatcher must reject the corresponding OP_TYPE or mode with a defined sticky error rather than ignoring it silently.

# Appendix C. Verification Evidence Checklist

Compiled RTL with no syntax errors.

All TC-CUR tests passed.

All claimed TC-PRI tests passed.

Assertions enabled during regression.

Golden model version recorded.

Known failing tests documented.

Waveforms captured for timing-sensitive blocks.

Performance counters dumped for demo cases.

Host raw tensor decode/NMS script tested with exported tensor format.

Final claim reviewed against evidence table.