EdgeASIC v5.0

RTL Implementation and Verification Specification

Module contracts, coding rules, test matrix, assertions, bring-up gates

Full-scope revision for YOLOv8n compute-layer execution

Revision 5.0  ·  17 August 2026  ·  Supersedes v4.8

# Document Control

| Field | Definition |
| --- | --- |
| Document | EdgeASIC v5.0 RTL Implementation and Verification Specification |
| Supersedes | EdgeASIC v4.8 RTL Implementation and Verification Specification |
| Revision | 5.0 |
| Date | 17 August 2026 |
| Purpose | Binding implementation and verification contract for every RTL module. Architectural rationale is in the companion Architecture Specification and is not repeated here. |
| Simulator | Icarus Verilog 12 (iverilog -g2012, vvp). Known limitations documented in §3.4. |
| Companion documents | EdgeASIC v5.0 Detailed Full-Scope Architecture Specification; EdgeASIC v5.0 36-Week Full-Scope Implementation Plan; EdgeASIC v5.0 Architecture Change Register |

# 1. Scope, Claims, and Implementation Discipline

## 1.1 Project definition

EdgeASIC is a host-assisted INT8 Conv/GEMM accelerator executing YOLOv8n compute layers. This document defines what each RTL module must do, how it is verified, and what evidence permits a claim. It is binding: a disagreement between this document and the RTL is a defect in one of the two and must be resolved, not tolerated.

## 1.2 Implementation classification

| Class | Meaning | Included items |
| --- | --- | --- |
| Verified | Implemented and passing a directed TC-ID as of 17 Aug 2026. | pe_mac, systolic_array_8x8, sddu, accum_engine, accum_buffer, requant, csr_block, descriptor_manager, operator_dispatcher, bank_state_controller, packing_fifo, axi_dma_master_abs, dma_read_router, activation_sram, weight_sram, output_sram |
| Scheduled | Assigned a week and a TC-ID in the 36-week plan. | conv2d_window_gen, tile_sequencer, weight_load_seq, lut_silu, maxpool_unit, elementwise_unit, descriptor_ring, tensor_table, graph_scheduler, c2f_sequencer, dma_arbiter, interrupt_logic, perf_counters, bias_quant_sram, residual_sram, edgeasic_top |
| Addressing | Realised in descriptor arithmetic; no module. | concat, split, upsample |
| Host | Software. | decode, NMS, confidence filtering, rendering |

## 1.3 Implementation discipline

| Item | Requirement |
| --- | --- |
| Test before merge | No RTL merges to main without at least one directed testbench or an explicit logged TODO. |
| TC-ID before claim | A feature without a passing TC-ID may not be described as implemented in any document or slide. |
| Golden comparison | Every datapath feature is accepted on bit-exact agreement with the Python model. Waveform inspection is diagnostic evidence only. |
| Branch and pull request | Mandatory. Direct file re-upload through the GitHub web UI is prohibited; it previously deleted five localparam definitions in config_pkg.sv silently and broke four verified modules. |
| Continuous integration | From Week 3, CI runs make all on every pull request and blocks merge on failure. |
| Package changes | config_pkg and types_pkg are jointly owned. Unilateral modification is prohibited. |

## 1.4 Version-to-version correction

| Version | Correction |
| --- | --- |
| v4.8 | Defined primary-target engines and the 65% scope trim. |
| v5.0 | (1) Target network changed to YOLOv8n. (2) Trim withdrawn; all engines scheduled. (3) lut_silu added to the drain path with a 256-entry SRAM table. (4) concat/split/upsample reclassified from engines to addressing and removed from the module list. (5) dma_arbiter, interrupt_logic, bias_quant_sram, residual_sram added. (6) Synthesis, area, power and utilization added to the verification scope. (7) Ownership rebalanced: Person A takes synthesis lint and critical-path work. |

# 2. RTL Module Hierarchy and Integration Strategy

Figure 1 — RTL module hierarchy. Green: verified as of 17 Aug 2026. Yellow: scheduled in the 36-week plan.

## 2.1 Top-level integration shell

edgeasic_top instantiates the control plane, compute core and memory system and exposes only the host CSR bus and the external AXI interface. No status or performance data may be read through top-level debug ports, because those ports do not exist in a packaged IP and any measurement taken through them is unreproducible.

## 2.2 Ownership split

| Owner area | Responsibility |
| --- | --- |
| Person A — datapath and verification | pe_mac, systolic_array_8x8, sddu, accum_engine, accum_buffer, requant, lut_silu, elementwise_unit, maxpool_unit, conv2d_window_gen, tile_sequencer, weight_load_seq; Python golden model and vector generation; synthesis lint and critical-path work. |
| Person B — control, memory and software | csr_block, descriptor_manager, descriptor_ring, tensor_table, graph_scheduler, c2f_sequencer, operator_dispatcher, interrupt_logic, perf_counters; all memory and DMA modules; host driver; regression infrastructure and CI; synthesis flow and reports. |
| Joint | config_pkg, types_pkg, edgeasic_top, and every integration week (1, 5, 7, 21–23, 29). |

## 2.3 Integration order

Compilation order is strict and enforced by the Makefile: packages first (config_pkg.sv then types_pkg.sv), then RTL dependencies in dependency order, then the testbench. Violating this order produces elaboration errors that are easy to misattribute to the design under test.

| Step | Integration | Rationale |
| --- | --- | --- |
| 1 | Window generator + tile sequencer + array | Establishes the operand feed before anything downstream can be trusted. |
| 2 | Add SDDU and accumulator | Deskew and accumulation are already verified in isolation; this step verifies their contract at the boundary. |
| 3 | Add requantisation and activation | Completes the drain path to INT8. |
| 4 | Add DMA and bank state | Closes the loop from external memory through compute and back. |
| 5 | Add descriptor ring and scheduler | Converts per-layer execution into network-level execution. |
| 6 | Add between-layer engines | Pooling and detect head operate on completed tensors and integrate last. |

# 3. Global Packages, Parameters, and Coding Rules

## 3.1 Required packages

| Package | Required content |
| --- | --- |
| config_pkg | N, ARRAY_N, PE_COUNT, DATA_W, ACT_W, WGT_W, OUT_W, ACC_W, ACC_BUFF, BIAS_W, SCALE_W, SHIFT_W, AXI_DATA_W, AXI_ADDR_W, ACC_ADDR_W, CHAN_W, TENSOR_ID_W, LUT_DEPTH, op_type_e, act_mode_e |
| types_pkg | int8_s, int16_s, int32_s, int33_s, int57_s, pipe_meta_t, descriptor_t, tensor_entry_t |

Package integrity requirement ARRAY_N, ACT_W, WGT_W, OUT_W and ACC_BUFF were previously deleted from config_pkg.sv by a web-UI file re-upload, breaking four verified modules with no visible diff. Mitigation: package files may only be modified via pull request; CI compiles all packages and runs the full regression on every PR; a package-level smoke test (TC-BOOT-001) verifies every required localparam is present and has its expected value.

## 3.2 Numeric widths

| Quantity | Width | Implementation rule |
| --- | --- | --- |
| Activation, weight | INT8 | Signed throughout. Never widen implicitly. |
| Product | INT16 | act × wgt inside the PE. |
| Column partial sum | INT32 | ACC_W. |
| Accumulator | INT33 | ACC_BUFF = 33. Per-sub-tile bound 8 × 128 × 128 = 131,072. The 33rd bit is load-bearing; 32 is unsafe across the full csr_k range. The golden model asserts this bound. |
| Bias | INT32 | BIAS_W. |
| Scale | 24-bit unsigned | SCALE_W, per output channel. |
| Shift | 8-bit unsigned | SHIFT_W. |
| Scaled product | INT57 | INT33 × 24-bit unsigned. |
| Output | INT8 | Saturating. |
| LUT depth | 256 entries | LUT_DEPTH = 2^OUT_W. See Architecture Specification Chapter 12. |
| AXI beat | 512-bit | AXI_DATA_W. |
| Accumulator address | ACC_ADDR_W = 8 | 256 rows. |

## 3.3 Dynamic pipeline metadata

K_TILE_FIRST and K_TILE_LAST are generated by the tile sequencer and travel as pipeline metadata. They must never be implemented as static descriptor bits. Metadata must arrive at each consumer on the same clock edge as the data it describes; the SDDU metadata pipeline depth must therefore equal MAX_DELAY exactly.

## 3.4 Coding rules and simulator constraints

- Use always_ff for sequential logic and always_comb for combinational logic. Do not mix.

- Use non-blocking assignment in sequential blocks and blocking assignment in combinational blocks, without exception.

- Every combinational block must assign every output on every path; unassigned paths infer latches.

- Use !== rather than != in testbench comparisons so that X and Z are detected rather than silently matching.

- Icarus Verilog 12 does not support break inside a function; restructure with an explicit done-flag loop.

- Icarus Verilog 12 does not support return inside a task; restructure with else-if nesting.

- Icarus Verilog 12 reports "constant selects in always_* processes are not currently supported" for certain part-selects and silently widens the sensitivity. Any module raising this warning must be re-checked against synthesis inference before it is claimed. requant.sv currently raises it and is scheduled for resolution in Week 25.

- Testbench vector arrays must be sized to the number of vectors actually generated. tb_requant_golden.sv currently requests [0:19999] from 5,000-word files, which is benign at the present VEC_BEATS but reads X if the parameter is raised; scheduled for Week 3.

- Elaboration errors are captured with 2>&1 | head -20 and pass/fail is determined by grepping for PASSED, FAILED and ERROR.

# 4. Control Plane Implementation Contract

## 4.1 CSR register map

| Offset | Register | Access | Definition |
| --- | --- | --- | --- |
| 0x00 | CONTROL | W | Bit 0 START (pulse), bit 1 SOFT_RESET, bit 2 PERF_CLEAR, bit 3 IRQ_ENABLE. |
| 0x04 | STATUS | R / W1C | Bit 0 BUSY, bit 2 sticky unsupported-op, bit 4 sticky AXI boundary, bit 6 sticky shape error, bit 8 sticky BSC error. |
| 0x08 | IRQ_STATUS | R / W1C | Done, error and performance-event flags. |
| 0x0C | IRQ_MASK | R/W | Per-source enable. |
| 0x10–0x2C | Descriptor window | W | A_BASE, W_BASE, O_BASE, BQ_BASE, INPUT1_BASE, LUT_BASE. |
| 0x30–0x38 | Shape window | W | M, N, K, KERNEL, STRIDE, PAD. |
| 0x3C | OP_CONFIG | W | OP_TYPE, ACT_MODE, residual_en, partial_tile_en, tensor_table_en. |
| 0x40 | DESC_COMMIT | W | Commit pulse; accepted only when the engine is not busy. |
| 0x44–0x4C | Ring control | R/W | RING_HEAD (host writes), RING_TAIL (hardware advances), RING_STATUS. |
| 0x50–0x5C | Tensor Table window | W | TT_INDEX, TT_BASE, TT_SHAPE, TT_STRIDE. |
| 0x60–0x74 | Performance counters | R | TOTAL, COMPUTE, DMA_WAIT, STALL, WRITEBACK, MAC_ACTIVE. |

## 4.2 Descriptor commit behaviour

| Topic | Requirement |
| --- | --- |
| Shadow buffer | The inactive descriptor mirrors host writes continuously; the active descriptor updates only on an accepted commit. |
| Safe-to-commit | A commit is accepted only when the engine is not busy. commit_accept pulses on acceptance. |
| Atomicity | The entire descriptor transfers in one cycle. Partial updates are prohibited. |
| Reset values | All fields zero except op_type = OP_CONV_GEMM and act_mode = ACT_NONE. |
| Ring interaction | When the ring is enabled, the scheduler supplies descriptors and direct CSR commit is ignored rather than silently racing. |

## 4.3 Tensor Table

| Requirement | Implementation rule |
| --- | --- |
| Entry format | tensor_id, base, shape (H, W, C), stride (row, channel), consumer count, valid. |
| Lookup | Combinational read on tensor_id; registered write. |
| Lifetime | Consumer count decrements on each consuming descriptor completion. A tensor is reusable only at zero. |
| Validity | Set by the producing descriptor completion. A consumer issuing against an invalid tensor stalls; it must not read. |
| Capacity | 16 entries (TENSOR_ID_W = 4). Exceeding capacity raises a shape error rather than wrapping. |

## 4.4 Graph scheduler

| Scheduler rule | Requirement |
| --- | --- |
| Issue policy | In-order only. Out-of-order and speculative issue are prohibited in this revision. |
| Dependency stall | A descriptor stalls until every source tensor is valid. |
| Branch | A tensor with multiple consumers is retained until the consumer count reaches zero. |
| Rejoin | A descriptor with two sources issues only when both are valid. |
| Deadlock guard | A timeout counter raises an error rather than hanging. Required because a Tensor Table defect otherwise presents as a silent hang. |
| Error halt | An error halts issue and preserves ring state for host inspection. |

# 5. Memory System Implementation Contract

| Memory block | Implementation requirement |
| --- | --- |
| activation_sram | Ping-pong banked, INT8 tiles, 512-bit write port from the DMA router, read port to the window generator. |
| weight_sram | Ping-pong banked, INT8 weights, read port to the weight-load sequencer. |
| output_sram | INT8 output tiles, write from the drain path, read to the packing FIFO. |
| bias_quant_sram | Bias, scale and shift indexed per output channel. Read by the accumulator (bias) and the requantiser (scale, shift). |
| residual_sram | Second-operand tensor for the elementwise unit. |
| lut_sram | 256-entry SiLU table. Written by DMA at layer start; read combinationally by the activation unit. |
| accum_buffer | 1R1W synchronous SRAM, ACC_BUFF × ARRAY_N wide, 2^ACC_ADDR_W deep. Synchronous read is mandatory; the earlier asynchronous model is not physically realisable. |

## 5.1 AXI boundary and response policy

| Condition | Required behaviour |
| --- | --- |
| Aligned request | Issue the AXI transaction and await response; assert dma_done on completion. |
| 4 KB crossing | Reject locally. Assert dma_done and dma_error together and set the sticky boundary error. No AXI transaction may be issued. |
| Boundary computation | Lower 12 bits of the address plus the byte count compared against 4096. Exact-boundary and off-by-one cases are covered by TC-DMA-004. |
| No silent drop | Every request completes exactly once, either normally or with an error. A request that neither completes nor errors is a deadlock defect. |

## 5.2 Bank State Controller invariants

- A bank is never simultaneously owned by the prefetch and compute paths.

- A bank in FILLING is never read by compute.

- A bank in VALID is never overwritten by prefetch.

- Simultaneous fill-done and consume-done on the same bank raises bsc_error and locks out rather than corrupting ownership.

- Reset leaves all banks EMPTY and free for prefetch.

## 5.3 DMA arbiter

| Rule | Requirement |
| --- | --- |
| Priority | Writeback takes priority over prefetch when the output path is at risk of backpressure stalling compute. |
| Starvation avoidance | A pending request may not be deferred indefinitely; an ageing counter promotes it. |
| Visibility | Arbitration-induced stalls are counted in DMA_WAIT so they are attributable during characterisation. |

# 6. Compute Datapath Implementation Contract

Figure 2 — Pipeline stages and v5.0 insertion points. Green: existing verified stages. Yellow: v5.0 additions.

## 6.1 Weight-stationary PE contract

| PE topic | Requirement |
| --- | --- |
| Weight hold | wgt_load captures wgt_in into wgt_reg. Computation uses wgt_reg, never wgt_in directly. |
| Signed arithmetic | act_in × wgt_reg is a signed INT16 product. All four sign combinations verified. |
| Registered act_out | act_out is registered so activation advances one column per enabled cycle. |
| Enable gating | With enable low, act_out and psum_out hold. No state advances. |
| clear_acc | Selects between product alone and psum_in + product. |

## 6.2 SDDU implementation

| Topic | Requirement |
| --- | --- |
| Delay profile | Lane c is delayed by MAX_DELAY − c enabled cycles, MAX_DELAY = ARRAY_N − 1. Lane ARRAY_N−1 is combinational passthrough. |
| Metadata depth | The valid, k_tile_first, k_tile_last and acc_addr pipelines are exactly MAX_DELAY deep, matching lane 0. |
| Enable freeze | With enable low the entire pipeline including metadata holds. Verified in TC-SDDU-005. |
| Valid window | out_valid is asserted for ARRAY_N cycles, delayed by MAX_DELAY enabled cycles from in_valid. |
| Bubble handling | A deasserted in_valid propagates as a masked output cycle with metadata zeroed. |

## 6.3 Accumulation engine

| K-tile case | Required behaviour |
| --- | --- |
| FIRST=1, LAST=1 | bias + MAC; drain to requantisation; no buffer access. |
| FIRST=1, LAST=0 | bias + MAC; write to buffer; no drain. |
| FIRST=0, LAST=0 | Read buffer, add MAC, write back; no drain. |
| FIRST=0, LAST=1 | Read buffer, add MAC, drain; no write. |
| RAW hazard | A read at an address written in the preceding cycles is served by the bypass path. Verified at maximum hazard pressure including a stall coinciding with a bypass cycle. |

## 6.4 Requantisation and activation

| Feature | Implementation rule |
| --- | --- |
| Product | INT33 accumulator × 24-bit unsigned scale into INT57. |
| Rounding | HALF_UP via the identity (x + 2^(s−1)) >> s == (x >>> s) + x[s−1], eliminating the 58-bit variable shifter. |
| Shift zero case | shift = 0 must be handled explicitly; the identity does not apply. |
| Saturation | Clamp to INT8 range. Wrapping is a defect. |
| Activation ordering | Applied after requantisation, without exception. |
| LUT-SiLU | 256-entry table indexed by the INT8 code. Table loaded by DMA at layer start from lut_base. Reload must not stall the pipeline mid-layer. |

## 6.5 Between-layer engines

| Engine | Implementation rule |
| --- | --- |
| maxpool_unit | Compare tree over the window. 2×2 and 5×5 supported. Operates on a completed tensor in SRAM, not on the drain stream. |
| SPPF | Repeated 5×5 pooling with concatenation of intermediates, sequenced by descriptors rather than by dedicated RTL. |
| elementwise_unit | Residual add with independent input scales; both operands rescaled to a common scale before addition, then saturated. |
| detect head | Standard convolution at three scales. Novelty is descriptor sequencing and the raw export layout, not datapath. |

# 7. Host/IP Synchronisation and Driver Contract

## 7.1 Host responsibilities

- Preprocess, quantise and tile tensors into the agreed NHWC layout with 64-byte row padding.

- Fold BatchNorm into per-channel scale and shift offline.

- Generate a 256-entry SiLU LUT per layer from that layer’s quantisation parameters.

- Populate the Tensor Table and emit the descriptor sequence for the network graph.

- Write CSR configuration, issue START, then poll DONE or wait for the interrupt.

- Read raw detection tensors and perform decode, confidence filtering and NMS.

## 7.2 IP responsibilities

- Execute the descriptor sequence without host intervention between layers once the ring and scheduler are enabled.

- Raise sticky errors for unsupported operations, shape errors and boundary violations before any engine starts.

- Maintain accurate performance counters for every cycle class.

- Never require a power-on reset to recover from an error it can raise.

## 7.3 CPU-only versus CPU+IP measurement

Both paths must produce equivalent detections on the same input before any timing is reported. Timing is then reported at two levels — kernel and full system — and never merged. The CPU baseline must be a controlled INT8 kernel performing the same operation, not an optimised framework performing a different one.

# 8. Reset, Errors, Interrupts, and Recovery

| Block | Reset requirement |
| --- | --- |
| All FSMs | Return to IDLE. |
| Bank state controller | All banks EMPTY, free for prefetch. |
| Accumulator buffer | Contents undefined; correctness must not depend on reset contents. |
| Performance counters | Zeroed unless a debug mode preserves them. |
| Sticky errors | Cleared on hard reset; preserved across soft reset so the host can diagnose. |
| Descriptor ring | Head and tail zeroed on hard reset; preserved on soft reset. |
| LUT SRAM | Contents undefined; must be reloaded before the first SiLU layer. |

| Error bit | Meaning |
| --- | --- |
| error_unsupported_op | Dispatcher received an OP_TYPE it cannot route. |
| error_axi_boundary | A DMA request would have crossed a 4 KB page boundary. |
| error_shape | M, N, K or a stride/padding combination is illegal for the selected operation. |
| bsc_error | Producer and consumer asserted completion on the same bank in the same cycle. |
| error_tensor_invalid | A descriptor referenced a Tensor Table entry that is not valid. |
| error_timeout | The scheduler stalled beyond the deadlock guard threshold. |

# 9. Performance Counters

| Counter | Increment predicate |
| --- | --- |
| TOTAL_CYCLES | Every cycle between START and DONE. |
| COMPUTE_CYCLES | Compute FSM active. |
| MAC_ACTIVE_CYCLES | At least one PE performing a useful multiply (excludes padding beats and masked lanes). |
| DMA_WAIT_CYCLES | Compute stalled awaiting a DMA response, including arbitration delay. |
| STALL_CYCLES | Compute stalled for any reason other than DMA. |
| WRITEBACK_CYCLES | Output path active. |

MAC_ACTIVE_CYCLES must exclude padding beats and masked lanes. If it does not, the utilization figure reported in Week 28 overstates efficiency, which is the single most consequential measurement defect available to this project.

# 10. Verification Environment Architecture

Figure 3 — Verification environment. The Python golden layer generates vectors; the SystemVerilog layer consumes them; the scoreboard attributes mismatches.

## 10.1 Verification layers

| Layer | Purpose |
| --- | --- |
| Python golden model | Independent bit-exact reference. 40 self-tests. Asserts the ACC_BUFF bound. |
| Vector generator | Randomised stimulus and expected outputs in $readmemh format, multi-seed. |
| Directed unit testbenches | Per-module corner cases and interface contracts. |
| Golden regression harness | Long randomised sweeps comparing RTL against the model beat by beat. |
| End-to-end testbench | Single-layer and multi-layer bit-exactness through the full path. |
| Network-level testbench | Backbone, neck and head per-layer comparison; full forward pass. |
| Scoreboard | Mismatch attribution to a specific lane, beat and stage. |

## 10.2 Python golden model contract

- The model is authoritative for arithmetic. Where RTL and model disagree, the model is presumed correct until proven otherwise.

- The model implements HALF_UP rounding, INT8 saturation and post-requantisation activation identically to the specification.

- The model asserts that no per-sub-tile accumulation exceeds the INT33 bound.

- Vector files must be generated with the same seed set recorded in the regression log so results are reproducible.

- Model and RTL must be updated in the same pull request when a shared behaviour changes.

# 11. Test Matrix

## 11.1 Verified test IDs (passing as of 17 Aug 2026)

| TC ID | Test name | Blocks |
| --- | --- | --- |
| TC-PE-001..007 | Signed MAC, weight stationarity, registered act_out | pe_mac |
| TC-ARR-001..008 | Array timing, sign combinations, psum seeding, weight corruption | systolic_array_8x8 |
| TC-SDDU-001..007 | Reset, delay, valid window, wavefront, enable freeze, bubble, metadata alignment | sddu |
| TC-ACC-001..004 | K-tile cases, RAW bypass, interleaved addresses, signed accumulation | accum_engine, accum_buffer |
| TC-ACC-ADV | Adversarial stall plus bypass plus overflow | accum_engine |
| TC-REQ-001..00n | Directed requantisation and activation | requant |
| TC-REQ-GOLD | 5,000-beat golden regression, 4,257 valid beats, 0 mismatches | requant |
| TC-CSR-001/002 | Reset values, sticky errors, W1C | csr_block |
| TC-DESC-001/002 | Shadow commit, busy interlock | descriptor_manager |
| TC-ERR-001 | Unsupported opcode routing | operator_dispatcher |
| TC-BSC-001/002 | Ownership transitions, collision | bank_state_controller |
| TC-FIFO-001/002 | Pack 8 B to 64 B, backpressure | packing_fifo |
| TC-DMA-001/002/004 | Normal transfer, boundary rejection, exact-boundary corner | axi_dma_master_abs |
| TC-DMA-003 | 512-bit beat lane mapping | dma_read_router |
| TC-MEM-001 | Write/read integrity | activation_sram, weight_sram, output_sram |

## 11.2 Scheduled test IDs

| TC ID | Test name | Week |
| --- | --- | --- |
| TC-WGEN-001/002 | Counter sequence; padding edges and corners | 1 |
| TC-SEQ-001 | K_TILE_FIRST/LAST placement | 1 |
| TC-E2E-001..004 | Single layer bit-exact; multi-K; negative; saturation | 1–3 |
| TC-CONV1X1-001..003 | 1×1 convolution modes | 2 |
| TC-STRIDE-001, TC-MASK-001 | Stride 2; partial tile masking | 2 |
| TC-REG-001/002 | Multi-seed randomised sweep | 3, 19 |
| TC-SILU-001..003 | All 256 codes; per-layer reload; mode switching | 4 |
| TC-CHAIN-001..003 | Two-layer handoff; five-layer CBS; backpressure | 5–8 |
| TC-RING-001..004 | Push/pop, full, empty, wrap | 6 |
| TC-PERF-001..004 | Counter correctness; full set; kernel and system comparison | 8, 18, 30 |
| TC-POOL-001/002, TC-SPPF-001 | 2×2 and 5×5 pooling; repeated pooling | 9 |
| TC-CAT-001, TC-SPLIT-001, TC-UP-001 | Addressing-only operator validation | 10 |
| TC-TT-001..003 | Tensor Table entry, decode, ID reference | 11 |
| TC-GS-001..005 | In-order, dependency stall, branch, rejoin, lifetime | 12–13 |
| TC-C2F-001..003 | C2f block, shape propagation, variable bottleneck count | 14 |
| TC-EW-001..003 | Residual add, mismatched scales, saturation | 15 |
| TC-HEAD-001..003 | Single and three scales; raw export layout | 16 |
| TC-ARB-001..003 | Priority, starvation, concurrency | 17 |
| TC-IRQ-001..003 | Done, error, W1C clearing | 18 |
| TC-SW-001, TC-NET-001..004 | Python full network; backbone, neck, head, full forward | 20–23 |
| TC-MAP-001/002 | mAP on COCO subset; path equivalence | 24 |
| TC-SYN-001..005 | Lint, synthesis, timing, gate count, post-synthesis equivalence | 25–26 |
| TC-AREA-001, TC-PWR-001 | Area breakdown; power estimate | 27 |
| TC-UTIL-001, TC-SCALE-001/002 | Per-layer utilization; 320×320; N=16 projection | 28 |
| TC-DEMO-001/002 | Path equivalence; recorded demo | 29, 36 |

## 11.3 Negative tests

- Illegal M, N or K values must raise a shape error before any DMA is issued.

- An unsupported OP_TYPE must not start any engine.

- A 4 KB-crossing request must not issue an AXI transaction.

- A descriptor referencing an invalid tensor must stall, not read.

- A commit attempted while busy must be rejected, not queued silently.

- A SiLU layer issued before its LUT is loaded must raise an error, not produce arbitrary output.

# 12. Assertion Plan

| Assertion ID | Property |
| --- | --- |
| A-PE-001 | With enable low, act_out and psum_out hold their previous values. |
| A-SDDU-001 | out_valid is asserted exactly MAX_DELAY enabled cycles after in_valid. |
| A-SDDU-002 | Metadata outputs change only on the same cycle as their associated data vector. |
| A-ACC-001 | k_tile_first and k_tile_last are never both asserted for a multi-sub-tile K unless K equals one. |
| A-ACC-002 | A buffer read never returns stale data when a write to the same address is in flight. |
| A-ACC-003 | No accumulated value exceeds the INT33 representable range. |
| A-REQ-001 | The output is always within the INT8 range. |
| A-REQ-002 | Activation is applied only after requantisation. |
| A-BSC-001 | A bank is never simultaneously VALID and FILLING. |
| A-DMA-001 | axi_valid is never asserted for a request flagged as crossing a 4 KB boundary. |
| A-DMA-002 | Every accepted request eventually asserts dma_done. |
| A-TT-001 | A tensor with a non-zero consumer count is never marked free. |
| A-GS-001 | A descriptor never issues while any source tensor is invalid. |
| A-GS-002 | The scheduler never stalls beyond the deadlock guard threshold without raising an error. |
| A-LUT-001 | The activation unit never reads the LUT before it has been loaded for the current layer. |
| A-PERF-001 | MAC_ACTIVE_CYCLES never exceeds COMPUTE_CYCLES. |

Icarus Verilog 12 has limited SystemVerilog assertion support. Properties that cannot be expressed as concurrent assertions in this simulator are implemented as procedural checkers inside the testbench, and the assertion ID is retained so that coverage of the property remains traceable.

# 13. Regression Strategy and Evidence

| Regression group | Contents |
| --- | --- |
| make core | pe, array, sddu, sddu_meta, accum, accum adversarial, requant, requant golden |
| make control | csr, descriptor manager, dispatcher, ring, tensor table, scheduler, C2f |
| make memory | bsc, packing FIFO, DMA master, router, arbiter, all SRAMs |
| make e2e | single-layer and multi-layer end-to-end bit-exactness |
| make net | backbone, neck, head, full forward |
| make golden | regenerate vectors and run all golden regressions |
| make sweep | multi-seed randomised regression |
| make all | everything above; run by CI on every pull request |

## 13.1 Required evidence per week

- Regression log showing pass or fail for every target.

- Waveform capture for every timing-sensitive change.

- Golden-model diff for any shared behaviour change.

- Updated TC-ID status in the claim register.

- Short note recording what passed, what failed, what is blocked.

# 14. Bring-Up Sequence and Integration Gates

| Gate | Name | Exit criterion |
| --- | --- | --- |
| G0 | Package integrity | All packages compile; every required localparam present with its expected value. |
| G1 | Operand feed | Window generator and tile sequencer produce the expected address and control sequence. |
| G2 | Single tile | One tile computes correctly through array, SDDU and accumulator. |
| G3 | Single layer | One conv layer bit-exact from a single descriptor. This is the Week 1 checkpoint. |
| G4 | Shape coverage | 1×1, 3×3, stride 1 and 2, partial tiles all bit-exact. |
| G5 | Activation | LUT-SiLU bit-exact across all 256 codes with per-layer reload. |
| G6 | Layer chaining | Five-layer CBS stack bit-exact with no host intervention. |
| G7 | Operator set | Pooling bit-exact; addressing operators validated. |
| G8 | Control plane | Ring, Tensor Table and scheduler drive a branching graph. |
| G9 | Full RTL | Every specification module implemented with a passing TC-ID. |
| G10 | Network | Full YOLOv8n forward pass on the IP. |
| G11 | Accuracy | mAP measured against the FP32 reference. |
| G12 | Physical | Synthesis, timing, area, power and utilization reported with tool evidence. |

# 15. Risk Register and Fallback Rules

| ID | Risk | Severity | Mitigation / fallback |
| --- | --- | --- | --- |
| R1 | Integration debug exceeds the Week 1 estimate | Critical | Week 1 checkpoint measures it directly; Weeks 2–3 are contingency; pre-approved fallback to trimmed scope plus addressing operators. |
| R2 | Silent package regression via web-UI upload | Critical | Branch and PR mandatory; CI blocks merge; TC-BOOT-001 verifies every localparam. |
| R3 | Control-plane phase overruns | High | Scheduler and C2f sequencing degrade gracefully to host-sequenced descriptors without invalidating the datapath. |
| R4 | requant.sv constant-select diverges in synthesis | Medium | Week 25 resolves the warning; Week 26 runs post-synthesis equivalence. |
| R5 | MAC_ACTIVE miscounts padding beats | Medium | Assertion A-PERF-001 plus a directed test comparing counter output against a hand-computed tile. |
| R6 | Deadlock in scheduler dependency tracking | High | Timeout guard raises an error; assertion A-GS-002; deadlock test with a deliberately unsatisfiable dependency. |
| R7 | Golden model and RTL drift apart | High | Model and RTL must change in the same pull request; CI runs both. |
| R8 | Vector array sizing reads X at higher VEC_BEATS | Low | Week 3 corrects the array bounds and adds a generation-count assertion. |

## 15.1 Fallback classification

If a scheduled module cannot be completed, it is reclassified at submission as implemented but partial, architecture-only, or host-side. It is never described as implemented. The fallback classification must appear in the architecture specification, the plan status table and the thesis simultaneously; inconsistency between them is a claim defect.

# Appendix A — Module Checklist

| Category | Modules |
| --- | --- |
| Packages | config_pkg, types_pkg |
| Compute core | pe_mac, systolic_array_8x8, sddu, accum_engine, accum_buffer, requant, lut_silu, elementwise_unit, maxpool_unit |
| Sequencing | conv2d_window_gen, tile_sequencer, weight_load_seq, c2f_sequencer |
| Control plane | csr_block, descriptor_manager, descriptor_ring, tensor_table, graph_scheduler, operator_dispatcher, interrupt_logic, perf_counters |
| Memory | axi_dma_master_abs, dma_arbiter, dma_read_router, bank_state_controller, packing_fifo, activation_sram, weight_sram, output_sram, bias_quant_sram, residual_sram, lut_sram |
| Top level | edgeasic_top, prefetch_fsm, compute_fsm, writeback_fsm |
| Removed from v4.8 module list | split_engine, concat_engine, upsample_engine — reclassified as addressing operations |

# Appendix B — Verification Evidence Checklist

- Every module in Appendix A has at least one directed testbench.

- Every datapath module has a golden-model comparison.

- Every timing-sensitive module has a captured waveform.

- Every negative test in §11.3 has a passing TC-ID.

- Every assertion in §12 is either a concurrent assertion or a procedural checker with a traceable ID.

- make all passes from a clean clone with no manual steps.

- Multi-seed sweep results are reproducible from the recorded seed set.

- Synthesis, timing, area and power reports are archived in syn/.

- The claim register maps every claimed feature to a passing TC-ID.

End of document — EdgeASIC v5.0 RTL Implementation and Verification Specification, 17 August 2026